SECTION 08 36 0

INSULATED SECTIONAL OVERHEAD DOORS

ArmRLite: CONTEMPO SMOOTH MODEL (CSM)

SPECIFIER NOTES should appear in Blue

# PART 1  GENERAL

## ATTN SPECIFIER: delete items below not required for project

1.1 	SECTION INCLUDES

- Glazed or Paneled Steel Sectional Overhead Doors

- Operating hardware and supports

## ATTN SPECIFIER: delete items below not required for project

1.2 	RELATED SECTIONS

- Section 03300 - Cast-In-Place Concrete: Prepared opening in concrete. Execution requirements for placement of anchors in concrete wall construction.

- Section 04810 - Unit Masonry Assemblies: Prepared opening in masonry. Execution requirements for placement of anchors in masonry wall construction.

- Section 05500 - Metal Fabrications: Steel frame and supports.

- Section 06114 - Wood Blocking and Curbing: Rough wood framing and blocking for door opening.

- Section 07900 - Joint Sealers: Perimeter sealant and backup materials.

- Section 08710 - Door Hardware: Cylinder locks.

- Section 09900 - Paints and Coatings: Field painting.

- Section 11150 – Parking Control Equipment: Remote door control.

- Section 16130 - Raceway and Boxes: Empty conduit from control station to door operator.

- Section 16150 - Wiring Connections: Electrical service to door operator.

1.3 	REFERENCES

A. 	ANSI/DASMA 102 - American National Standard Specifications for Sectional Overhead Type Doors.

1.4 	DESIGN / PERFORMANCE REQUIREMENTS

## ATTN SPECIFIER: delete items below no required for project

A. 	Wiring Connections: Requirements for electrical characteristics.

  - 115 volts, single phase, 60 Hz

  - 230 volts, single phase, 60 Hz

  - 230 volts, three phase, 60 Hz

  - 460 volts, three phase, 60 Hz

1.5 	SUBMITTALS

- Submit under provisions of Section 01300.

- Shop Drawings: Indicate plans and elevations including opening dimensions and required tolerances, connection details, anchorage spacing, hardware locations, and installation details.

- Manufacturer's Certificates: Certify products meet or exceed specified requirements.

- Operation and Maintenance Data.

1.6 	QUALITY ASSURANCE

- Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with minimum (20) years documented experience.

- Installer Qualifications: Authorized representative of the manufacturer with minimum (10) years approved by manufacturer.

1.7 	DELIVERY, STORAGE, AND HANDLING

- Store products in manufacturers’ unopened labeled packaging until ready for installation.

- Protect materials from exposure to moisture until ready for installation.

- Store materials in a dry, ventilated, weathertight location.

1.8 	WARRANTY

A. 	5-year warranty on sections – standard 1-year warranty on remaining parts.

# PART 2  PRODUCTS

2.1 	MANUFACTURERS

A. 	Acceptable Manufacturer: For purposes of designating type and quality of work under this Section, Drawings and Specifications are based on products manufactured or Furnished by ARMRLITE DOOR

MANUFACTURING CO. Toll Free: 800-554-5816  P:908-754-2600 ext 4016 Shannon McGrady F: 908-754-6522 email: smcgrady@armrlite.com W: www.armrlite.com.

- Registered WBE-Women owned business enterprise.

- Substitutions:  Not permitted.

2.2 	INSULATED SECTIONAL OVERHEAD DOORS

A. 	Insulated Sectional Overhead Doors: CONTEMPO SMOOTH MODEL (CSM) by ArmRLite Door Manufacturing Co., Inc. Units shall have the following characteristics:

1. 	Door Assembly: Foamed in place construction – CFC free.  Smooth and flush exterior with Pebble grain texture interior.

- Section Thickness: 1-3/4 inches

- R-Value 16

i. 	Springs:

## ATTN SPECIFIER: delete items below not required for project

- 10,000 cycles

- 25,000 cycles

- 50,000 cycles

- 75,000 cycles

- 100,000 cycles

j. Lites:

ATTN SPECIFIER: delete items below not required for project. Please consult our Architectural Liaison Department for extended glass options beyond what is listed below. P: 908-754-2600 E: smcgrady@armrlite.com.

- (__) 24” x 12 lites per section

- (__) 24” x 8 lites per section

- Full vision section(s) to contain 1/8” non-insulated glazing

- Full vision section(s) to contain 1/4” non-insulated glazing

- Full vision section(s) to contain 7/16” insulated glazing

- Full vision section(s) to contain 1” insulated glazing

- Full vision section(s) to contain 9/16” hurricane glass

2. 	Finish and Color:

- Custom Finish exterior per architect supplied sample. (consult factory regarding custom finish colors).  Interior standard white

- Hardware: All hinges and fixtures made of 14-gauge galvanized steel. Full floating, ball bearing rollers to have hardened steel races and roller sizes to be adequate for design requirements and limitations. Heavy-duty, fully adjustable roller brackets are attached to each end cap to provide an easy adjustment of the door to the job for proper seal. All hardware is heavy-duty and rust resistant with galvanized fasteners.

## ATTN SPECIFIER: delete items below not required for project

- Stainless Steel Corrosion Resistant Package: Stainless steel track, face hardware, and cables for high moisture or corrosive environment.

- Powdercoat Corrosion Resistant Package: Powdercoated track and face hardware to provide added corrosion resistance to galvanized steel.

- Lock: Interior galvanized single unit.

- Weatherstripping:

  - Bottom section astragal.

  - Jamb seals.

  - Header seal.

- Track: Provide track as recommended by manufacturer to suit loading required and clearances available.

## ATTN SPECIFIER: delete items below not required for project

- Low Headroom Track

- Standard Lift Track

- High lift Track

- Full Vertical Lift Track

- Roof Pitch Track

## ATTN SPECIFIER: delete items below not required for project

- Manual Operation: pull rope

- Manual Operation: chain hoist

- Electric Motor Operation: provide electric operator, size and type as recommended by manufacturer.

a. 	Entrapment Protection: Required for momentary contact, includes radio control operation. b. 	Operator Controls:

1) 	Push-button operated control stations with open, close, and stop buttons.

- High Performance Electric Motor Operation: ArmRSpeed high performance electric operator.

18” – 25” per second open/close speed. Subject to weight and headroom limitations; please consult ArmRLite prior to specifying

# PART 3  EXECUTION

3.1 	EXAMINATION

- Verify that all openings are ready to receive work and opening dimensions and tolerances are within specified limits.

- Beginning of installation means acceptance of existing surfaces.

  - PREPARATION

- General Contractor to prepare opening per ArmRLite shop drawings. All steel reinforcing or wood blocking for track, springs and operator mounting by General Contractor.

3.3 	INSTALLATION

- Install door unit assembly in accordance with manufacturer’s instructions.

- Anchor assembly to wall construction and building framing without distortion or stress.

- Secure tracks to structural members only.

- Fit and align door assembly including hardware, level and plumb, to provide smooth operation.

- Install perimeter trim and closures

3.4 	TOLERANCES

- Maintain dimensional tolerances and alignment with adjacent work

- Maximum variation from plumb: 1/8 inch

- Maximum variation from level: 1/8 inch

- Longitudinal or diagonal warp: plus or minus 1/8 inch from 10 ft. straight edge

3.5 	CLEANING AND ADJUSTING

- Clean doors, frames and glass.

- Remove labels and visible markings.

3.6 	PROTECTION

- Do not permit construction traffic through overhead door openings after adjustment and cleaning.

- Protect installed products until completion of project.

- Touch-up, damaged coatings and finishes and repair minor damage before Substantial Completion.

END OF SECTION 08 36 13